Apex Chassis Stabilizer Clamp Install Instructions

INSTALLATION:

- Place clamp in best position for stabilizer per manufacturer’s instructions.

- Use C-Clamp tool to tighten halves together around drag link or tie rod.

  - NOTE: By design, clamp halves will not contact each other on both sides.

  - Be sure to test lock-to-lock steering motion to ensure proper range of stabilizer.

  - Check for contact with other vehicle components, correct if required.

- Install retaining bolts using a blue medium strength thread locker.

  - DO NOT OVERTORQUE these fasteners, they will strip out.

  - Torque to 8 FT/LB or 100 IN/LB.

  - Follow torque sequence below.

- Install set screws to prevent the rod from spinning in the clamp.

Trust Apex Chassis for quality parts that stand the test of time!