Toyota 4Runner Tie Rods (TR230) Installation Instructions:

- Start by measuring where the center of the ball stud connects to the knuckle then to end of the inner tie rod that connects to the rack and write down for use later.

- Disconnect outer tie rod from knuckle first.

- Slide the bellows down and off so you can access the inner joint with an adjustable wrench (36mm or WR103) and remove from rack.

- Grab the new APEX tie rod assembly (TR230) and disassemble.

- Thread the inner tie rod into the steering rack and tighten to 60 ft-lbs. (36mm or WR103)

- With the inner joint tightened, slip bellows over the top of the steering rack and install with supplied clamp.

- Install the outer clamp with the boot in the groove of the tie rod.

- Thread outer tie rod to inner tie rod and adjust to measurement you took earlier

- Install the outer tie rod end into the knuckle and tighten castle nut. Torque castle nut to 67 ft-lbs NOTE: The Apex Chassis stamp should be facing right side up on outer tie rod.

- Install cotter key pins. If you can’t see through castle nut and stud keep tightening until cotter key pin can pass through.

- We recommend using medium strength thread locker (blue Loctite 243 or blue Permatex 24206 or other preferred brand) under the jam nuts, then tighten jam nut(19.5mm) to 60 ft/lbs.

- Install the grease zerks. Our tie rods come pre-greased, however a small amount can be added if necessary, DO NOT OVERGREASE

- Repeat process on other side

- At this point we highly recommend engaging a professional to get the alignment set.

- Check boots and jam nut torque at oil change intervals

If you have any questions or concerns, please don’t hesitate to give us a call or email.

Thank you,

Team Apex

sales@apexchassis.com

480-470-5500